# Modular 6-DOF Atmospheric Trajectory Simulator

System Specification, Architecture, and API

# Purpose and Design Philosophy

This document defines the architecture and implementation plan for a modular six-degree-of-freedom (6-DOF) rigid-body trajectory simulator operating primarily within the atmo-sphere.

The simulator is designed to:

- Support free-fall objects, rockets, and simple aircraft

- Use quaternion-based rotational dynamics

- Separate numerical computation (C++) from orchestration and visualization (Python)

- Support control, guidance, and Monte-Carlo simulation

- Remain extensible to future physics models (e.g., rarefied flow) The design strictly separates:

- Dynamics solver

- Physical models

- Control and guidance logic

- Visualization and analysis

# Scope Definition

## In-Scope

- 6-DOF rigid-body translational and rotational dynamics

- Quaternion attitude propagation

- Aerodynamic force and moment modeling

- Wind modeling

- Thrust modeling (optional)

- Time-varying mass

- Control and guidance interfaces

- Monte-Carlo batch simulation

- Python-based visualization

## Out-of-Scope (Current)

- Flexible-body dynamics

- Structural deformation

- Thermal modeling

- Real-time hardware-in-the-loop

# Coordinate Frames

## Inertial Frame (I)

Earth-fixed NED or abstract Cartesian inertial frame. Used for position, velocity, and wind defini-tion.

## Body / Control Frame (B)

Origin at center of mass. Axes:

• +X forward (nose)

• +Y right

• +Z upward

Used for state propagation, thrust, control, and moments.

## Aerodynamic Frame (A)

Origin at nose. Axes:

• +X backward (drag direction)

• +Y right

• +Z downward

Used for aerodynamic coefficients.

# State Definition

## Core State Vector

- Position rI = [x, y, z]

- Velocity vI = [vx, vy, vz]

- Quaternion qBI

- Angular velocity ωB = [p, q, r]

- Mass m(t) (optional)

## Derived Outputs

Euler angles (visualization only), speed magnitude, aerodynamic angles.

# Inputs

## Aerodynamic Coefficients

- Force: Cx, Cy, Cz

- Moment:  CMx, CMy, CMz

Defined in the aerodynamic frame.

## Control Inputs

Four abstract channels:

- p (roll)

- q (pitch)

- r (yaw)

- i (brake / drag)

Control meaning is defined inside coefficient models.

## Atmosphere

Built-in model or CSV versus altitude. Wind is defined in inertial frame.

## Thrust

Optional CSV thrust versus time, applied in body frame.

# Dynamics Model

## Translational

r˙I

= vI

v˙ I

1

= m FI

## Rotational

˙	1

q = 2 Ω(ωB)q

Iω˙ B + ωB × (IωB) = MB

# Force and Moment Evaluation Pipeline

- Compute relative wind velocity

- Transform velocity to body frame

- Transform to aerodynamic frame

- Evaluate aerodynamic coefficients

- Compute aerodynamic forces and moments

- Transform to body frame

- Add thrust and gravity

# Control and Guidance Architecture

Control logic conforms to:

u = f (x, t, environment)

The solver is control-agnostic. Guidance, control laws, and observers are implemented exter-nally.

# Monte-Carlo Simulation

Monte-Carlo simulations are executed in Python by randomizing inputs and repeatedly invoking the same C++ solver.

# Software Architecture

## Language Responsibilities

| Layer | Language | Responsibility |
| --- | --- | --- |
| Dynamics core | C++ | Integration, forces, frames |
| Models | C++ / Python | Aero, atmosphere, thrust |
| Orchestration | Python | Configuration, execution |
| Visualization | Python | Plotting and animation |

- C++ Code Structure and File Responsibilities

cpp/

math/ frames/ dynamics/ models/ integrators/ api/

| File / Folder | Description |
| --- | --- |
| math/vector.hpp | Basic 3D vector operations |
| math/quaternion.hpp | Quaternion algebra and normalization |
| frames/transform.hpp | Frame transformation utilities |
| dynamics/rigid body.cpp | 6-DOF equations of motion |
| integrators/rk4.cpp | Time integration methods |
| models/aero model.hpp | Aerodynamic coefficient interface |
| models/atmosphere.hpp | Atmosphere and wind models |
| models/thrust.hpp | Thrust interpolation logic |
| api/simulator api.cpp | Python-callable simulation interface |

# Python Code Structure and File Responsibilities

sim/

config/ models/ control/ guidance/ monte_carlo/ visualization/ runner/

| File / Folder | Description |
| --- | --- |
| config/vehicle.py config/mission.py models/coefficient models control/controller.py guidance/guidance.py monte carlo/run.py visualization/plots 2d.py visualization/trajectory 3 runner/single run.py | Vehicle physical properties Initial conditions and simulation settings .Apyerodynamic coefficient logic Control laws (PID, etc.) Reference trajectory generation Batch simulation manager Time-history plotting d3.Dpytrajectory visualization Single simulation execution |

# Execution Flow

- Load configuration files

- Initialize atmosphere, vehicle, and models

- Initialize state vector

- For each time step:

  - Query atmosphere and wind

  - Evaluate guidance and control logic

  - Compute aerodynamic coefficients

  - Compute forces and moments

  - Integrate state in C++

- Store simulation history

- Visualize results

- Appendix A: Python–C++ API

  - C++ State Structure

struct State { double p o s I [ 3 ] ; double v e l I [ 3 ] ;

double quat BI [ 4 ] ; double omega B [ 3 ] ; double mass ;

} ;

  - C++ Control Input

struct Control Input {

double p ; double q ; double r ; double i ;

} ;

  - C++ Solver Interface

extern ”C” void s t e p s i m u l a t i o n ( State ∗ s ta te ,

const Control Input ∗ c o n tr o l ,

double time ,

double dt

) ;

## Python Usage Example

for k in range (N) :

s t e p s i m u l a t i o n ( s ta te ,  c o n tr o l , t , dt ) h i s t o r y . append ( copy ( s t a t e ) )

t += dt

# Appendix B: Detailed Function-Level Interface Specification

This appendix defines the function-level interfaces for all C++ and Python scripts in the simulator. The intent is to provide a clear contract between modules while allowing internal implementation flexibility.

  - C++ Core Library Functions

### math/vector.hpp

      - Vector3 add(const Vector3& a, const Vector3& b)

        - Input: two 3D vectors

        - Output: vector sum

        - Description: Basic vector addition

      - Vector3 cross(const Vector3& a, const Vector3& b)

        - Input: two 3D vectors

        - Output: cross product

        - Description: Used for rotational dynamics

      - double dot(const Vector3& a, const Vector3& b)

        - Input: two 3D vectors

        - Output: scalar dot product

        - Description: Used for projections and norms

### math/quaternion.hpp

      - Quaternion normalize(const Quaternion& q)

        - Input: quaternion

        - Output: normalized quaternion

        - Description: Prevents numerical drift

      - Quaternion multiply(const Quaternion& q1, const Quaternion& q2)

        - Input: two quaternions

        - Output: quaternion product

        - Description: Frame composition

      - Vector3 rotate(const Quaternion& q, const Vector3& v)

        - Input: quaternion, vector

        - Output: rotated vector

        - Description: Frame transformation

### frames/transform.hpp

      - Vector3 inertialToBody(const State& state, const Vector3& v)

        - Input: state, inertial vector

        - Output: vector in body frame

        - Description: Uses quaternion for frame rotation

      - Vector3 bodyToAero(const Vector3& v)

        - Input: body-frame velocity

        - Output: aerodynamic-frame velocity

        - Description: Applies aerodynamic frame convention

### models/atmosphere.hpp

      - AtmosphereState  query(double  altitude)

        - Input: altitude

        - Output: pressure, density, temperature, wind

        - Description: Interpolates built-in or CSV atmosphere

### models/aeromodel.hpp

      - AeroCoefficients evaluate(const AeroInput& input)

        - Input: Mach, Reynolds, body rates, control inputs

        - Output: force and moment coefficients

        - Description: User-defined aerodynamic coefficient logic

### models/thrust.hpp

      - Vector3 thrustAtTime(double t)

        - Input: simulation time

        - Output: thrust vector in body frame

        - Description: Interpolates thrust CSV

### dynamics/rigidbody.cpp

      - void computeDerivatives(const State& state, Derivative& dstate)

        - Input: current state

        - Output: state derivatives

        - Description: Implements 6-DOF equations of motion

### integrators/rk4.cpp

      - void integrate(State& state, double dt)

        - Input: state, time step

        - Output: updated state

        - Description: Fourth-order Runge–Kutta integrator

### api/simulatorapi.cpp

      - void step simulation(State* state, const ControlInput* control, double time, double dt)

        - Input: mutable state pointer, control input, time, timestep

        - Output: updated state

        - Description: Single-step simulation entry point for Python

## Python Orchestration Functions

### config/vehicle.py

      - load vehicle(config file)

        - Input: configuration file

        - Output: vehicle object

        - Description: Loads mass, inertia, geometry

### config/mission.py

      - load mission(config file)

        - Input: configuration file

        - Output: initial state and time settings

        - Description: Defines initial conditions

### models/coefficientmodels.py

      - evaluate coefficients(state, atmosphere, control)

        - Input: state, atmosphere, control

        - Output: aerodynamic coefficients

        - Description: Python-side coefficient model

### control/controller.py

      - compute control(state, t)

        - Input: state, time

        - Output: ControlInput object

        - Description: Implements control laws

### guidance/guidance.py

      - reference(state, t)

        - Input: state, time

        - Output: reference commands

        - Description: Generates guidance targets

### montecarlo/run.py

      - run ensemble(N)

        - Input: number of runs

        - Output: statistical results

        - Description: Executes Monte-Carlo simulations

### visualization/plots2d.py

      - plot history(time, states)

        - Input: time array, state history

        - Output: figures

        - Description: Time-history plots

### visualization/trajectory3d.py

      - plot trajectory(states)

        - Input: state history

        - Output: 3D visualization

        - Description: 3D trajectory rendering

### runner/singlerun.py

      - run()

        - Input: none

        - Output: simulation results

        - Description: Executes a single simulation case

# Appendix B Summary

This appendix establishes a complete function-level contract between all modules, ensuring that each component can be independently developed, tested, and extended without modifying the core solver.

# Conclusion

This document defines a complete, extensible, and performance-oriented 6-DOF trajectory simula-tion framework suitable for research, control development, and Monte-Carlo analysis.